# Neural Network

MDE - Advanced Engineering Topics, ITESO

Gomez Enriquez, Gerardo

gerardo.gomez@iteso.mx

Guadalajara, Mexico

# Implementation choices

| Design space | Implementation choice | Trade-offs | Choice benefit | Choice downside |
| --- | --- | --- | --- | --- |
| Datapath reutilization | Time-multiplexed | Area / Throughput | Less area | Less throughput |
| Internal accumulation style | Parallel multipliers with CSA tree (FMA DP) | Area / Latency / Timing / Complexity | Less latency, area & complexity than pipe. Better timing than chain | More area than chain |
| Multiplier vector width | N = 8 | Area / Throughput / Latency | Balanced area for target performance | One image every 149 cycles |
| Combinational / Pipelined | Combinational | Area / Throughput / Latency | Less area & latency | Less throughput |

# Area / resource usage analysis

## FMA dot-product

### Operands width

Source A: 8-wide vector of 9-bit operands

- Multiplexed between: Pixel (4-bit) and hidden neuron activation (8-bit) + 1 bit to force unsigned / positive value

Source B: 8-wide vector of 8-bit weights (signed)

Source C: Single 32-bit operand

- Multiplexed between: Bias and accumulator (both widths 32-bit)

### Internal structure

- 8 multipliers in parallel inside FMA dot-point (no FMA chain)

  - 1 Wallace tree per multiplier (each multiplier output is 2 operands)

  - 1 Wallace tree to reduce all multipliers “partial results”

  - 1 Carry Look-Ahead Adder as final CPA

| Description | Instances | Operands in | Operands out | Levels |
| --- | --- | --- | --- | --- |
| Per-multiplier Wallace tree | 8 (one per multiplier) | 6 each (5 Booth PPs + 1 two's-comp correction) | 2 each | 3 CSA |
| Final Wallace tree | 1 (shared) | 17 (8×2 mult outputs + 1 srcc) | 2 | 6 CSA |
| Final CPA adder (CLA) | 1 (shared) | 2 (32-bit) | 1 (32-bit result) | 3 LCU |

### Area breakdown

| FP DP Item | LUTs |
| --- | --- |
| Per multiplier (without Wallace tree) | ~71 (70–73) |
| Wallace tree instance per multiplier | ~69 (64–75) |
| Final Wallace tree | 633 |
| Final CPA(CLA) | 134 |
| Total | 1,893 |

### LUT usage comparison with different numbers of multipliers (N)

| N | Operands into final tree (2·N+1) | Final tree CSA levels (≈log₁.₅) | Multipliers LUTs (≈141×N) | Final Wallace tree LUTs (≈42.2×(ops−2)) | CLA LUTs (fixed) | Total LUTs (≈) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 1 | 141 | 42 | 134 | ~317 |
| 2 | 5 | 3 | 282 | 127 | 134 | ~543 |
| 4 | 9 | 4 | 564 | 295 | 134 | ~993 |
| 8 | 17 | 6 | 1,126 | 633 | 134 | 1,893 |
| 16 | 33 | 7 | 2,256 | 1,308 | 134 | ~3,698 |
| 64 | 129 | 11 | 9,024 | 5,359 | 134 | ~14,517 |

- These are approximations, not real synthesis for N != 8.

- Area scales near-linearly with N. However the final Wallace tree grows logarithmically, which is the non-linear part of the estimate.

## FF usage

| Signal | Bits | Comments |
| --- | --- | --- |
| acc | 32 |  |
| hidden_act | 128 | Hidden neuron layer activations (16 × 8) |
| argmax | 32 |  |
| predicted_digit | 4 |  |
| delayed_fma_dp_result | 32 | Registered fma_dp_result, decouples argmax compare from the FMA critical path |
| delayed_output_neuron_en | 1 | Registered output_neuron_en (paired with the delayed result) |
| delayed_sel_output_neuron | 4 | Registered sel_output_neuron (paired with the delayed result) |
| control_fsm.state | 4 | One-hot, re-encoded from 2-bit enum (4 states: IDLE, LAYER1, LAYER2, DONE) |
| control_fsm.counter_passes | 4 | Pass counter within a neuron (Max count = 8) |
| control_fsm.counter_neurons | 5 | Neuron counter (0–25 total neurons) |
| control_fsm.counter_weights | 8 | Weight-ROM address counter (0–147) |
| weights_rom | 351 | Implemented with LUTs (asynchronous read). Quartus inserted these registers during physical synthesis |
| biases_rom | 0 | Implemented with LUTs (asynchronous read). |
| RTL total | 254 |  |
| Synthesis report total | 619 |  |
| Gap | 365 | Number differs due to physical-synthesis optimizer: register retimer, duplication and removal |

## Resource usage summary

| Component | LUTs | FFs |
| --- | --- | --- |
| weights_rom | 1,131 | 351 |
| biases_rom | 16 | 0 |
| control_fsm | 58 | 23 |
| argmax_cmp | 32 | 0 |
| fma_dp | 1,893 | 0 |
| neural_network_digits (own logic) | 420 | 245 |
| Total | 3,550 | 619 |

- The weights ROM dominates area (32% of the design) expected, since it's implemented as logic-based ROM due to asynchronous read requirements that dedicated memories in the FPGA does not support.

- The weights ROM output has very high fanout. Each of the 64 weight bits feeds into 8 parallel Booth-multiplier instances, each with multiple partial-product terms consuming that same bit. A wide, deep async-read mux tree driving that much fanout is the kind of shape that the physical-synthesis optimizer looks for register retiming and duplication for fanout as a legal way to shorten combinational delay or reduce net loading

- The net effect of the 351 registers Quartus added to weights_rom are not extra logical state. They're the tool exploiting otherwise-idle flip-flops already present in the LUT-based ROM's Logic Elements to help meet the timing target.

# Timing analysis

## Critical path

From counter_weights[*] → To hidden_act[*][*] — 26 logic levels, slack +0.419 ns

| Description | Node | Count |
| --- | --- | --- |
| Weight ROM address decode | weights_rom¦rom_data~696/697/698¦combout | 3 |
| Booth PP-select mux (multiplier) | gen_mul[2]...¦Mux28~0/1¦combout | 2 |
| Per-multiplier Wallace tree | gen_mul[2]...¦wallace_tree¦gen_lvl[0..2].gen_csa[0].csa¦...¦cout/sum | 3 |
| Final Wallace tree | fma_dp¦wallace_tree¦gen_lvl[0..5].gen_csa[*].csa¦...¦cout/sum | 6 |
| Carry look-ahead Adder | fma_dp¦cla¦gen_lvl[0..1].gen_lcu[*].lcu4¦gg/c | 7 |
| Wide-OR reduction (clamp check in requantization) | WideOr0~1/2/3/6¦combout | 4 |
| Hidden-activation writeback mux | hidden_act_nxt[0][3]~7¦combout | 1 |
| Total |  | 26 |

- The path traverses the entire dot-product engine end-to-end: ROM address decode → Booth PP mux → per-multiplier CSA reduction → shared final CSA reduction → CLA → requantization (WideOr, the clamp-to-255 check) → hidden-activation register.

- The earlier structural analysis that stacked 3 (per‑mul CSA) + 6 (final CSA) + 3 (CLA LCU) levels dominates the critical path along with the asynchronous ROM read and requantization.

- In red, the critical path.

- In blue, the registered fma_dp_result (delayed_fma_dp_result), decouples argmax compare from the FMA critical path. Before delaying the signal that was the critical path. The delay was engineered to avoid affecting the final latency of the design.

## Timing metrics summary

| Metric | Value |
| --- | --- |
| Target frequency | 50.0 MHz (20.000 ns period) |
| Fmax (Slow 1200 mV, 85 °C) | 51.07 MHz |
| Worst setup slack | +0.419 ns (met) |
| Max LUT depth | 28 |
| Latency | 149 cycles |
| Throughput | 1 image every 149 cycles |

- Timing closure was achieved at the 50 MHz target with positive slack (+0.419 ns), and the design's Fmax (51.07 MHz) sits just above that target.

- The optimization mode was set to high performance effort to achieve this frequency, otherwise the maximum frequency was 49 MHz.

- The retiming and register-duplication optimizations seen in weights_rom were the mechanisms Quartus used to reach this narrow positive slack at 50 MHz since the asynchronous ROM read sits in the critical path of the design.

- In an attempt to improve timing, the Wallace tree was optimized by removing the Wallace trees per multiplier and adding the partial products from all multipliers together in a single Wallace tree. The number of CSA levels decreased by one but somehow timing got worse and the change was reverted.

# Conclusion

The chosen implementation is not well suited for high-performance systems: its combinational datapath limits the maximum frequency, and the resulting throughput is low. It does, however, offers a reasonable balance between area and throughput/latency for applications that do not demand high-rate image processing.