# Saturation

MDE - Advanced Engineering Topics, ITESO

Gomez Enriquez, Gerardo

gerardo.gomez@iteso.mx

Guadalajara, Mexico

# Architecture and considerations

This is an adder with saturation that can saturate to a maximum and a minimum configurable / dynamic value. Both saturations can be enabled/disabled with another set of signals.

The adder used is a Carry Look-ahead Adder (CLA) and the saturation logic is in series with the result and carry-out. The saturation logic consists of signed and unsigned comparators that check the configured saturation values with the adder result (concatenated with its carry-out for complete adder precision). The final output result is muxed depending on the overflow / underflow detection and the saturation enablement.

Since the saturation values are configurable / dynamic the overflow and carry-out output signals from the adder cannot be used directly with as in other adder with saturation architectures.

# Area / resource usage analysis

## LUT usage

| Width | Adder LUTs | Saturation Logic LUTs | Total LUTs | Saturation Logic % |
| --- | --- | --- | --- | --- |
| 8 | 37 | 56 | 93 | 60.20% |
| 16 | 69 | 104 | 173 | 60.10% |
| 53 | 244 | 326 | 570 | 57.20% |
| 128 | 599 | 776 | 1,375 | 56.40% |

- The cost of the saturation logic in this architecture is higher than the adder (CLA) logic itself in all width configurations.

- The saturation logic consistently accounts for ~57–60% of the total LUTs, with the fraction slightly decreasing at wider widths. The CLA grows linearly with width, and its two comparators drive the saturation cost

# Timing / latency analysis

## Fmax & Max logic depth

| Width | Fmax (MHz) | Max LUT Depth |
| --- | --- | --- |
| 8 | 174.25 | 8.5 |
| 16 | 135.34 | 11.1 |
| 53 | 101.32 | 15 |
| 128 | 71.68 | 23.1 |

- The saturation logic makes heavy use of arith cells. Quartus weighs arith-mode LUT cells (carry-chain adder cells) at 0.5 depth instead of 1.0, since they use dedicated carry hardware rather than the LUT fabric.

- Both Fmax and logic depth scale as expected with width. The fractional depths reflect the mixed arith + normal LUT path.

## Critical paths

The critical path is the same for all widths: src*_q[*] → result_q[*]. It goes through the same logic elements, summarized below for the different width implementations.

| Logic description | Node | W8 (LoL=8.5) | W16 (LoL=11.1) | W53 (LoL=15) | W128 (LoL=23.1) |
| --- | --- | --- | --- | --- | --- |
| CLA — LCU carry-generate/propagate | cla¦gen_lvl[*].gen_lcu[*].lcu4¦gg/c¦combout | 3 | 5 | 8 | 11 |
| CLA — FA propagate | cla¦gen_fa[*].fa_gp¦p¦combout | — | 1 | — | — |
| CLA — FA sum | cla¦gen_fa[*].fa_gp¦sum¦combout | 1 | 2 | 1 | 1 |
| Comparator carry chain | LessThan*~N¦cout → ~M¦combout | 2 | 2 | 16 | 27+ |
| Saturation select | sel_max/min_sat_value~0¦combout | 1 | 1 | 1 | 1 |
| Result MUX | result[*]~N¦combout | 2 | 2 | 2 | 1 |

Notice that the logic levels are weighted by Quartus differently for arithmetic more logic such as the dedicated hardware for carry chain (LessThan*~N|cout).

## Other timing parameters

- Latency: 1 cycle (single cycle)

- Throughput: 1 operation per cycle

- Slack: Significantly negative. Frequency was set to 1 GHz for analysis purposes and getting the maximum possible frequency, which was 174.25 MHz when width=8.

# Conclusion

The CLA contributes a roughly logarithmic number of logic levels as width grows, consistent with its look-ahead carry structure. The saturation comparator, however, grows linearly because $signed(ext_result) > $signed(ext_max_sat_value) synthesizes as a ripple-carry chain over WIDTH+1 bits with no look-ahead.

The saturation logic (two WIDTH+1-bit comparators plus the output MUX) is structurally heavier than the CLA in terms of area. Each comparator requires roughly as many cells as a ripple-carry adder of the same width, so two comparators alone already outweigh the CLA in LUT count.

Replacing the ripple-carry comparators with CLA-style comparators — or reusing the adder's own carry and overflow flags to detect saturation — would recover both Fmax and a substantial fraction of the LUT budget.